| Hazard and Harm | Where or what task | Significant Yes/No | Action | Does the action eliminate, isolate, or minimise the risk? | How often is the action monitored | Date of last review |
| --- | --- | --- | --- | --- | --- | --- |
| Noise levels of machinery and equipment- risk of hearing loss | Working with and near noisy machinery and equipment such as granulators | Yes Contact OSH if hearing loss occurs | 1.For staff who work with or around noisy machinery and equipment: -Provide and maintain earmuffs -Ensure staffs wear earmuffs as required. 2. Regularly maintain equipment and machinery. | Minimise |  |  |
| Slips, trips and falls- risk of bruising, lacerations and broken bones | All work areas | Yes Contact OSH if serious injury occurs | 1.Train staff in safe work practices including to: -use ladders and platforms where necessary -clean up spills immediately - keep work area tidy and uncluttered -wear non-slip footwear 2. use the handrails that have been installed on stairways 3.maintain ladders and platforms | Minimise |  |  |
| Manual handling- risk of muscles strain and back pain | All tasks that require moving and lifting heavy loads | Yes Contact OSH if person is hospitalised for 48hours, within 7 days of harm occuring | 1. Train staff in reducing the load: - Use trolleys, pallet jacks and fork lift to move heavy loads - store heavy loads down low -organise work tasks so the loads are lifted between mid thigh and shoulder height -use ladders rather than reaching above shoulders -keep loads close to the body - use team lifting for heavy objects | Minimise |  |  |
| Hazard and Harm | Where or what task | Significant Yes/No | Action | Does the action eliminate, isolate, or minimise the risk? | How often is the action monitored | Date of last review |
| Occupational Overuse Syndrome- risk of damage to muscles, tendons etc | All repetitive tasks | Yes Contact OSH if serious injury occurs | 1.Work station ergonomic design 2. Training in technique and posture 3. Rest periods and stretching 4. Early reporting to be encouraged | Minimise |  |  |
| Chemical reactions- risk of poisoning & Skin damage | All tasks requiring use of chemicals especially at printing station | Yes Contact OSH if serious harm is caused | 1. Follow suppliers instructions. 2. Do not smoke or consume food whilst working with chemicals. 3. Store properly and keep secure 4. Employees are not to use hazardous substances unless they have been trained in their use or are under direct supervision. 5. where appropriate employees will be advised of the procedures to be followed if spillage of hazardous substances occurs. | Minimise |  |  |
| Transportation- Risk of motor vehicle accident | Working off site -for example delivering product | Yes Contact OSH if serious accident occurs | 1.Only appropriately qualified staff can drive/operate vehicles 2. All vehicles are to be maintained in good repair and not to be driven when known to be unsafe 3. All freight should be properly secured 4.Trailers are to be securely fixed to the towing vehicle and stored safely when not in use. | Minimise |  |  |

| Hazard and Harm | Where or what task | Significant Yes/No | Action | Does the action eliminate, isolate, or minimise the risk? | How often is the action monitored | Date of last review |
| --- | --- | --- | --- | --- | --- | --- |
| Moving parts of machinery- risk of crushing or entrapment | All tasks involving direct contact with machinery | Yes Contact OSH if serious accident occurs | 1.Do not wear loose clothing or untied long hair when working on site 2. Only properly trained/experienced staff are permitted to use hazardous equipment, unless they are under direct supervision. 3.Guards must always be in place when operating 4.Regular maintenance of machines | Minimise |  |  |
| Working with hot materials- risk of burns | All tasks requiring handling of hot materials | Yes contact OSH if serious injury occurs | 1.Always wear gloves when handling hot materials | Minimise |  |  |
| Working with sharp objects or cutting device- risk of piercing or cutting | All tasks requiring cutting of materials | Yes contact OSH if serious injury occurs | 1.At no time should hands be placed in cutting area 2.If cutting devise becomes jammed, turn off the power supply to the unit before removing blocking object 3.Guards must always be in place when machine is operating 4.Always cut away from oneself | Minimise |  |  |

| Hazard and Harm | Where or what task | Significant Yes/No | Action | Does the action eliminate, isolate, or minimise the risk? | How often is the action monitored | Date of last review |
| --- | --- | --- | --- | --- | --- | --- |